SatSpy, Version 4.0

User's Manual

September 1, 2000

Acappella Publishing

Copyright (c) 1995-2000, All Rights Reserved

Introduction

Satellite viewing can be an interesting and rewarding hobby.  How is it that we can we see satellites?  A satellite is visible from the ground when the sun has set and it is dark where you are, but the satellite, flying high overhead, is still in bright sunlight.  What you are seeing is the sunlight reflecting off the satellite.

A satellite is nothing more than an artificial moon and just like the earth's natural moon, a satellite has "phases".  A satellite is brightest when the sun is to your back as you are looking toward the satellite.  This is the same geometry that makes the moon "full" once a month.  When the sun has just set and you are looking for a satellite in the eastern portion of the sky, you are looking toward its brightest side.   The same is true in the early morning except that you want to look to the west before the sun rises.

There are many good satellites to look for, but the best are the manned satellites such as the Russian space station (Mir), the International Space Station (ISS) and the US space shuttle.  These are some of the largest artificial satellites and also the brightest.  You will have no problem seeing any of these with your naked eye.  There are many other interesting satellites to see such as weather satellites, earth resources satellites and communications satellites.

All you really need to watch satellites is SatSpy and your eyes (and possibly a pair of binoculars).  SatSpy is a complete application that generates "passes" to show you when a satellite is visible.  The pass is projected against the sky as it appears from your latitude and longitude.  The pass projection includes all of the bright stars, constellations and planets.  This powerful feature makes it much easier for you to find the satellite you are looking for.  SatSpy also computes the correct lighting conditions, indicating how bright a satellite should be and how long it should be visible.

This User's Manual will give you an overview of how to use SatSpy.  SatSpy contains an extensive on-line help system which is the primary documentation.  For detailed information on how to perform specific operations, please consult the integrated help files by hitting the F1 key whenever you have a question.

SatSpy Basics

SatSpy consists of a single main window and multiple child windows which are opened within the main window.  Each child window contains graphical or tabular data representing items such as a satellite's pass or an element set summary.  The child windows are opened from commands on the main menu or from the toolbar.  Actions specific to each window are invoked using the "popup menu" which appears when you click on the right mouse button when the cursor is positioned within the child window.  Each of the interface elements is described in the figure below:

Running SatSpy

1	Click the Start button, and then point to Programs.

2	Click SatSpy to open the program group.

3	Click the SatSpy program icon to start.

Note:  When you start SatSpy for the first time, you will be prompted to enter information describing your viewing location.

To exit SatSpy, choose File| Exit from the main menu or use any of the standard Windows techniques for exiting a program.

Using the Main Menu

The main menu sits across the top of the main window.  Clicking on any one of the menu items brings up sub-menus from which you can select specific commands.  Many of these commands open child windows while others are used to bring up dialogs that allow you to set parameters to control SatSpy's data processing.

Using the Main Toolbar and its Floating Toolbars

The toolbar is the row of buttons just below the main menu.  It provides shortcuts to many of the common menu commands.  Each button maps to a main menu command.  Clicking on any of the toolbar buttons performs the same action as the corresponding menu command.  To see what a button does, position the cursor over a button and hold it steady for a half-second or so.  A tool tip showing the button's function will open and stay open as long as the cursor is held over the button.

The toolbar contains several smaller "floating toolbars".  You can move these away from the main toolbar by clicking on the line above the buttons and then dragging the floating toolbar while continuing to hold the mouse button down.  When you release the button, the floating toolbar will appear as 'mini-window' containing control buttons.  You can move this window around as desired.  To re-attach a floating toolbar to the main toolbar, click on the mini-window's caption bar and drag the floating toolbar above the main tool bar.  When you release the button, the floatting toolbar will become part of the main toolbar again.  You can toggle the floating toolbars off and on from the View item on the main menu.  This menu also gives you access the Zoom and Sky Orientation toolboxes which are not visible by default.

Accessing Popup Menus

SatSpy child windows let you use the right mouse button to open context a popup menu.  This menu contains commands that are applicable to the current window. To access this menu, click the right mouse button within the window (not including the title bar or edges).  The popup menu will appear on the screen, anchored at the point at which you click the right mouse button.

Using the Status Panel

The status bar is shown in light gray along the bottom of the SatSpy main window.  It has three areas. The first is used to display a clock.  By default, this shows the current local date and time.  You can change the display format by double clicking on the face of the clock.  The second area is the tip area.  Each time you click on or pass over a menu item, either from the main menu or from a pop-up menu, a line describing that item appears in the tip area.  The tip line gives you an indication of what the menu item does.  The third area shows the current viewing location.  To change the current default location, double-click to open the Set Your Viewing Location dialog.

Online Help

SatSpy contains an extensive on-line help feature using the Windows Help system.  In addition to using most of the standard help features, SatSpy Help provides the following capabilities:

Online Tutorial - The tutorial gives a quick introduction to SatSpy so you can get up and running as rapidly as possible.  The tutorial is intended to be used by running it concurrently with SatSpy.  You can run the tutorial from the SatSpy program group or from SatSpy's Help menu.

Context Sensitive Help - You can obtain help on the window you are currently using by clicking on the button labeled "?" in the toolbar or by pressing F1.  Many of the SatSpy dialog boxes have a button labeled "Help".  Pressing this button or the F1 key will bring up help specific to that dialog box.

Using SatSpy

Loading and Managing Element Sets

SatSpy operates on element sets (elsets) that you load into memory from files downloaded from the Internet or an online service such as CompuServe or America Online.  The element sets are used with one of three orbit propagators to model the motion of satellites.  The orbit propagators are discussed in more detail in subsequent sections.   By default, SatSpy uses SGP, the simplest and most efficient of the three.  If you wish to use a different propagator, you must specify it in the SatSpy Preferences dialog (Elsets page) before loading elsets.

To load a new element set file, select File|Load New Elsets from the main menu.  You can append additional element sets by selecting File|Append Elsets.  In either case, a dialog box will open prompting you to select an elset file.  Pick the file you are interested in and then click on the OK button.  When the elsets begin to load, the viewing location area changes to a progress bar along with a cancel button.  Pressing cancel stops the elset load operation.  If you select Load New Elsets, any elsets currently in memory will be deleted before loading new ones.  If you have an windows open which are using one or more elsets, you will be prompted to close the windows before loading new elsets.  You can append new elsets without closing any open windows.

Once you have loaded a collection of elsets into memory, you can append new elsets to the list as described above, clear the elset list, delete selected elsets or save the entire list to a new text file.  To clear the elset list, select File|Clear Elset List from the main menu.  To delete selected elsets, choose File|Delete… to open the Delete Elsets from Memory List dialog.  This dialog shows a tabular summary of all elsets currently loaded into memory.  You can sort the list on any column by clicking on one of the column header buttons.  Clicking the same button two times in a row changes the sort order.  This is useful in finding elsets which share similar features (e.g. periods greater than 225 minutes).  To delete one or more elsets, select them using the mouse and click the delete button.  You can delete as many elsets as you want.  When you are done, click Close to close the dialog.  If you would like to save the current elset list to file, select File|Save to File from the main menu and specify the name you want the elsets saved under.

If you are a registered user of SatSpy, you will also find the SatSpy Utilities helpful in managing your elset.  The Utilities are available under the File|Utilities menu and allow you to do things such as merge and filter elset files, extract elsets and visual magnitudes from Molczan files and convert between two line and three line elset format.  The filter function is especially useful if you have a subset of satellites that you regularly watch.  This function allows you define the set of satellites that you are interested in and then extract those elsets from a larger file.

For more information, see:

- Elsets Preferences

- Loading Elsets from a File

- Checksum - Describes checksum processing to insure elset integrity.

- Clearing the Elset List

- Deleting Selected Elsets

- Saving Elsets to a Text File

- Loading vs. Opening Elset Files

- Elset Filter Page

- Using the SatSpy Utilities

Set Your Viewing Location

Before you can compute viewing opportunities, you must first tell SatSpy your viewing location.  Do this by selecting Options|Set Viewing Location… from the main menu to open the Viewing Location Dialog from which you set the latitude, longitude and altitude of your geographic position.  You also need to provide time zone information so that SatSpy can convert the results of its internal time calculations to local times.  This information is associated with a location name which you either provide or pick from a list that SatSpy provides.  There is only one default location, but you can have multiple windows open at any time, each with a different viewing location.

SatSpy retains the current viewing location in memory.  Each time you open a new window that needs a viewing location, the current viewing location is 'attached' to the window.  Many windows, such as Skytrace and Viewing Opportunities, give you the option to select a viewing location from a drop-down list.  The current default viewing location is always shown when first opening these windows.  You may select a different location from the list.  When you do, the resulting window is shown from the perspective of that viewing location.  On some windows, such as the Local Sky and Horizon View, you can change the viewing location even after the window is open.  Select Controls|New Location from the popup menu to view the location list dialog.  Select the desired location from the list and click OK to redraw the window showing the sky as seen from the new location.

You can edit the contents of the viewing location list by selecting File|Location List from the main menu to open the Manage Locations List dialog.  The list box on the left shows the available viewing locations.  The list on the right shows the currently selected viewing locations.  Locations are moved between lists using standard Windows drag and drop techniques.  When you drop a new location into the selected list, click on the location name and verify that the time zone setting is correct and change it, if necessary, from the drop down time zone list.  The two check boxe at the bottom allow you to toggle the display of the location list symbols and names on the various SatSpy map windows.

TIP:  You can delete items from the selected location list by draging them on to the trashcan symbol (under the list) and then dropping.

For more information, see:

- Viewing Location Dialog

- How SatSpy Uses the Viewing Location

- Location List Dialog

- Using Multiple Viewing Locations

How to Use Animation in SatSpy

All graphical windows can be animated.  To initiate animation, open the Animation dialog by selecting Animate… from the popup menu or by using the Ctrl-A accelerator keys in any graphical window (except Viewing Opportunities).  Both the interval step and the time increment can be set from this dialog.  The interval step determines how frequently the window is redrawn.  The time increment is the amount of time (seconds, minutes, hours or days) added to the current time to get the new time. For skytrace windows, you should use a time increment of 5-15 seconds.  If you want to see the motion of the planets over a long period of time, use a time increment of 1-10 days.  The buttons along the bottom of the dialog are used to start, stop and pause animation.

You can also animate any window so that the window time is updated from your system clock.  This shows you how a satellite is moving in real time.   Since SatSpy is getting the time from your computer's clock, it is very important to ensure that your clock is correct.  To animate in real time, check the box labeled "R/T" on the Animation dialog.  This feature is especially useful in real time observing using a laptop computer and in monitoring missions such as the US Space Shuttle.

Most graphical windows give you the opportunity to open "child" windows using the View command from the popup menu.  For example, you can open a Skytrace and the select View|3D Orbit View to open a globe window showing the same pass in 3D view.  If you then begin animating the Skytrace window, you will notice that the 3D Orbit View is also updated immediately after Skytrace.

See also:

- Setting Your System Clock

- Animating Graphical Windows

- Using the Animation Dialog

Zooming and Panning to Find Details

All of the graphical windows can be "zoomed" to show finer details.   Once zoomed, you can pan the image left or right or up or down.  You can zoom using the Zoom Palette (floating toolbox) or the keyboard.  To use the Zoom Palette, select Zoom from the popup menu or View|Zoom from the main menu to open the palette.  The Zoom Palette works on the currently active window.  If you have multiple windows open, be sure to click on the window of interest to activate it before clicking on the palette buttons.

To zoom in or out, use the two leftmost buttons.  The button with the '+' zooms in while the button with the '-' zooms out.  The four arrow buttons to the right of the zoom buttons allow you to pan a zoomed window.   The next button to the right puts you into "zoom window" mode.  This mode allows you to zoom in on a specific rectangular region using the mouse to click and drag to the region of interest.  The last button on the right restores a zoomed window to its original unzoomed  state.

You can also zoom windows using shortcut keys.  The following short cuts work on the active window:

- Ctrl-I - Zoom In

- Ctrl-U - Zoom Out

- Up Arrow - Pan up

- Down Arrow - Pan down

- Left Arrow - Pan left

- Right Arrow - Pan right

Note:  Panning the 3D Orbit View window works slightly differently than the other windows.  For details, see the online help.

Customizing the Display Colors

SatSpy allows you to customize the colors of most items in the graphical windows. Colors are set according to window class, of which there are three:  Local Sky, Orbit Map and Viewing Opps.  The Local Sky class includes all of the windows that contain star maps.  The Orbit Map class includes the 3D Orbit View and Groundtrace windows.  The Viewing Opps. class consists of only the Viewing Opportunities window.  When ever you make changes to colors in a class, all window types in that class are affected.

To change colors, select Options|Preferences from the main menu.  This opens the SatSpy Preferences dialog, from which you choose the Colors page.  The page consists of three data groups:  the window class selection buttons, the display item selection list and the color controls panel.  The class selection buttons let you choose which group of windows that you want to change.  The item selection list shows the interface items that can be changed in each class.  When you select an item, the color controls panel shows you the current setting for that item.  You can change an item by clicking on the new color in the color array panel.   If the display item is a fill color, you can also change color by manipulating the red, green and blue components using the slide bars.

After you make your changes, click on the Show button to see the changes reflected on any windows which are currently open.  If you don't like the changes, click on the Cancel buttons to revert to the settings when you started.  If you have made changes and would like to return to the settings that ship with SatSpy click on the Default buttons.  If you would like to use the night sky color scheme, click on the Night Sky button.  This set of colors is for the star windows and is designed for out door viewing to preserve your night eyes for satellite watching.

For more information, see:

- Changing the Color Scheme

- SatSpy Preferences

How SatSpy Uses Time

The time that you see on all SatSpy dialogs and windows is the local civil time at the coordinates you specify in the Viewing Location dialog.  This convention allows you to set your watch to local time when looking for a satellite. You don't have to mentally convert from Universal Time Coordinated (UTC)  to your local time when observing satellites.

SatSpy automatically converts the local time from your system clock or from any input dialog to UTC for its internal calculations.  UTC can be thought of as local time at Greenwich, England.  By tradition and international agreement, UTC has become the "standard" time system.  It is sometimes, though incorrectly, referred to as Greenwich Mean Time (GMT).  The epoch time contained in a satellite's two-line element set is given in terms of a year, day and fractional day in UTC.  SatSpy performs all orbit propagation and pass generation using UTC (and an internal epoch of January 0.0 1980).  When displaying results, the UTC time is converted back to local time.

How SatSpy Models the Motion of Satellites

SatSpy uses element sets (elset) and orbit propagators to model the motion of satellites.  An elset is a collection of parameters that describe a satellite's orbit.  An orbit propagator is a mathematical model that simulates the motion of a satellite using an elset.  SatSpy uses three different orbit propagators:

- Two Body - This is the mathematically simplest propagator.  This should NOT be used for visual observations.  It is provide for advanced users only.

- SGP - This propagator models only those orbital perturbations necessary to give reasonably accurate predictions for near-earth satellites.  It is optimized for speed and should be used as the default propagator for all routine observations.

- SGP4 - Models additional gravitational and atmospheric terms and is more accurate for near-earth satellites.  The increased accuracy, however, comes at the cost of increased computational load and consequently slower performance.  This propagator should only be used when doing more precise analysis.

- SDP4 - Extends the SGP4 model to include additional perturbations affecting deep-space satellites (by definition, those with periods greater than 225 minutes).  These include lunar-solar gravitation and solar pressure effects.

All three propagators are public domain implementations of standard US government models supplied by Dr. T. S. Kelso of AFIT.  The element set data used by SatSpy is publicly available and released through NASA's OIG website.

SatSpy's Earth model uses the WGS 84 constants.  These define the shape of the earth and the gravitational constants used by the orbit propagators.  Specifically, the Earth is modeled as an oblate spheroid (kind of a squashed globe) with an equatorial radius of 6378.135 km and a flattening factor of 1/298.26.  For astronomical calculations, SatSpy uses the J2000 epoch for computing the positions of the Sun, Moon, planets and stars.

When comparing the output from SatSpy with similar programs, realize that different programs use different models for the Earth,  different constants, different propagators and astronomical models.  While these programs are internally consistent, they may produce results that are slightly different than those from SatSpy.  During testing, I have compared SatSpy output with a number of other models and found reasonable agreement with all.  The ultimate test, however, is its ability to predict satellite passes.

Trouble Shooting

What happens if you are unsuccessful at seeing a satellite?  This can happen for a number of reasons.  The most common is problems with your viewing location and time zone settings.  These are set in the Viewing Location dialog (select Options|Set Viewing Location from the main menu).  You can check your approximate viewing location by opening a Groundtrace window.  Your viewing location will be plotted on the map.  If your location is off by just a few kilometers, you should be OK for all but the most precise observations.  There are many resources available on the Internet to find your precise location.  Here are a few:

- Geographic Name Search (http://164.214.2.59/gns/html/index.html)

- US Census Gazetteer - (http://www.census.gov/cgi-bin/gazetteer)

- USGS GNIS Data Base Query (http://www-nmd.usgs.gov/www/gnis/gnisform.html)

- Canadian Geographical Names (http://geonames.nrcan.gc.ca/english/Home.html)

- Australian Geographic Place Names (http://kaos.erin.gov.au/database/MAN200R.html)

Once you have verified your viewing location, make sure that your time zone is set correctly.  A good way to do this is to compare the sun rise and set times predicted by SatSpy with those printed in your local newspaper.  The SatSpy predictions should be within a few minutes of those in your paper.  If the times are off by an hour or more, your settings are probably wrong. (A word of caution:  rise and set times vary with longitude.  If you live a long way from where your paper is printed, the comparison may be invalid).  SatSpy does not automatically update times for daylight savings time.  If your locality uses this convention, you will need to adjust your time zone information manually.

Working with Star Windows

Several of the SatSpy windows share a common attribute, the display of a star background.  All of these windows also share common functionality such as the ability to use the mouse to identify celestial objects, to change the orientation of the star background and to control which celestial objects are displayed.  Windows in this group include the Single Skytrace, the Multiple Skytrace, the Geosynchronous Satellites,  the Local Sky, the Local Horizon and the Right Ascension/ Declination windows.  The first four windows are local polar windows meaning that they show the sky as it appears from your latitude and longitude.  The last windows are a Cartesian plots, the first showing the local sky and the second showing the entire sky (the Celestial Sphere).

Identifying Celestial Objects

You can use the mouse to identify celestial objects including stars, planets and constellations.  Each window can be placed into a specific "find mode" using the popup menu.  All six windows share find modes for planets, stars and constellations.  The Multiple Skytrace window has modes to find pass details and satellite names.  The Single Skytrace also has modes to find ephemeris point details and to find and drag time labels for individual points.  The Geosynchronous Satellites window has modes to find satellite names and ephemeris point details.

To set the find mode, select the appropriate mode from the popup menu.  For example, in the Local Sky window, select Find|Star from the popup menu.  The shape of the cursor will change to indicate the find mode.  In this case, it shows a small circle with the word "Stars" to the lower right.  In planet and constellation modes, the words "Planet" and "Constl.", respectively, appear.  To find the identity of a particular star, move the cursor so that the small circle is over the star you are interested and then press and hold the left mouse button.   After a moment, the identity of the star will appear, along with the visual magnitude of the star.  The find planet and find constellation modes work in a similar fashion.  To find the identity of a constellation, you need to place the cursor over one of the stars at the ends of the connecting lines.

Changing the Sky Orientation

By default, the local sky plots are oriented with North at the top and east on the left of the window.  This is the preferred orientation for the printed page, but may not be your preferred viewing orientation.  You can change the orientation three ways:  1) by rotating the position of North, 2) by changing the East-West positions or 3) by choosing one of four pre-defined positions (N, S, E and W).  All three options are available from the Sky Orientation toolbox which can be opened from the popup menu on any local sky window or from the main menu by selecting View|Sky Orientation.

To rotate the position of North, press the "Rotate sky window" button (second from the right on the Sky Orientation toolbox).  The cursor will change shape to indicate that you are now in "rotation mode".  You will also see a line with a handle going from the center of the display towards the current North position.  Use the mouse to drag the handle (click and hold anywhere in the small circle) to a new position.  When you release the handle, the sky will be redrawn with North in the direction of the handle.  When you are done, press the "Rotate sky window" button again to reset the rotation state.

To change the East-West positions, click on the "E|W" button (farthest right on the Sky Orientation toolbox).  This will cause the local sky window to be redrawn as if it is reflected through an imaginary vertical line through the center of the window.  Note that this change will affect all open local sky windows.

Controlling What is Displayed

The Celestial Toolbox dialog is used to determine which objects (stars, planets, etc.) are displayed.  To open the Celestial Toolbox dialog, select Celestial Toolbox from the popup menu or press the F4 function key in any window showing stars.  From here, you can toggle the display of (any or all) the sun, moon, planets, constellations and the ecliptic.  You can also set the minimum magnitude for stars.  Only those stars with magnitude equal to or less than (brighter) the selected values are displayed. You can choose to display labels for the constellations, stars, solar system objects (sun, moon, planets) or stellar magnitudes for the brighter stars.  The buttons at the top of the dialog let you control zooming and panning as described above.

Tip:  You can shrink the size of the toolbox so that only the zoom/pan buttons are showing by double-clicking on the title bar.  Double-clicking a second time restores the toolbox to full size.

Day, Night and Twilight

The background color for all of the local sky windows changes depending on the elevation of the sun relative to the local horizon.  If the sun is above the horizon, the background color is light blue.  When sun is more than 18 degrees below the horizon (the end of astronomical twilight), the sky is black.  When the sun is between 0 and 18 degrees below the horizon, the color of the sky progresses from lighter to darker shades of blue to indicate the degree of twilight.

Note:  These are the default colors.  The colors can be changed as described in "Customizing the Display Colors".

Computing Satellite Passes

The purpose of SatSpy is to compute passes that let you predict where and when to look to see a satellite.  To do this you must first obtain fresh element sets, load them into SatSpy, compute one or more satellite passes, label the graphical pass, print it and then, go outside and watch the satellite fly over!  This chapter shows you how to accomplish these tasks.

The Single Skytrace Window

The Single Skytrace shows the path of a satellite as it moves across the sky as seen from your viewing location. This is the primary window that you will use to observe individual satellites.  It consists a circular plot showing the sky as seen from your viewing location.  The stars, planets and constellations are shown at the time corresponding to the mid-point of the satellite's pass.  An azimuth/elevation grid is superimposed on the image to help you find the direction to the satellite.   The satellite's path is indicated by colored line going across the sky.  The direction of motion is indicated by small arrows.  Sunlit portions of the path are shown in green.  When the path color switches to red, this means that the satellite has moved into the earth's shadow and is no longer visible.  You can find the time when the satellite passes a particular point by clicking on a tic-mark and dragging the mouse to a convenient location.  When you release the mouse, you will see the time and estimated magnitude (brightness) of the satellite at that point.

To open a new Single Skytrace window, select Sat Views|Skytrace|Polar... from the main menu or use the toolbar.   This will open the Compute Skytraces for Selected Satellites dialog, from which you will pick one or more satellites along with a time span of interest.  You can change the viewing location by selecting from the list of available locations in the drop-down list above the list of satellite names.  You can also select groups of related satellites by checking the group name in the scroll box just below the location names.  When you click OK, SatSpy computes the passes for the satellite(s) and you will see the Satellite Passes Dialog, from which you pick the pass you want.  This dialog gives a one line summary for each pass computed.   You can sort the pass list by clicking on any of the header buttons.  To pick a pass and open the Single Skytrace window, either select it using the mouse and click on the OK button or double-click the summary line.

Once you have opened a Single Skytrace for the pass of interest, use the time labeling feature to identify and tag specific points on the pass.  When you are ready to go outside to view the pass, you can make a hard copy of the window by selecting Print… from the popup menu (or from the main menu or toolbar).  If you choose the "East-West" orientation from Options|Viewing…, you can hold the print out against the sky for a better representation of the sky as you will see it when looking for the pass.

For more information on using the Single Skytrace window see the following topics in the online help:

- Single Skytrace

- Single Skytrace Detail

- Compute Skytraces Dialog

- Sky Orientation

- Using the Time Labels

- Estimating Visual Magnitudes

- Animating Graphical Windows

- Skytrace Ephemeris Data

- Satellite Passes Dialog

================================================================

8/17/2000 -  Stopped here.

================================================================

The Multiple Skytrace Window

Where the Single Skytrace window shows you the skytrace for one pass at a time, the Multiple Skytrace  window shows you all passes found over a specified time span.  Each satellite's pass is shown as a single skytrace projected against the local sky.  The positions of the planets, stars and constellations are shown at the mid-point of the time window.  The individual skytraces are color coded in the same fashion as the Single Skytrace window.  The traces also contain arrows indicating the direction of motion.  This window is great for planning your viewing sessions.  If you have a laptop computer, you can animate the window, in real time, to show the positions of the satellites as they are moving across the sky.

To open a new Multiple  Skytrace window, select Sat Views|Multiple Skytraces… from the main menu or use the toolbar.   This opens the Compute Multiple Skytraces dialog, from which you pick one or more satellites along with a time span of interest.  You can use the Filter button to apply constraints to the passes you wish to view.  When you click OK, the progress bar opens in the lower right-hand side of the main window and indicates the progress of the computations.  When complete, you see the Multiple Skytraces window open showing the passes available during the time span.  If no passes were found, you see an local sky plot showing without any trace lines.

Once you have opened the window, you can use the mouse to find more information on individual passes.  By default, the window's Find mode is set to Detail which means that when you click on a tic mark for an individual pass, you will see the Viewing Opportunity Detail dialog when you click.  This dialog shows a summary of the pass and has buttons along the bottom from which you can open a Single Skytrace, 3D Orbit View or Groundtrace window.  You can also open a Satellite Element Set Details dialog to learn more about the satellite.

When you select a large number of satellites for processing or a relatively large time span, the number of passes may be such that it is hard to distinguish individual passes.  Alternatively, you may be interested only in passes that go through a certain constellation.  In either case, you can use the Cone Filter to reduce the amount of visible passes to a more manageable number.  This filter allows you to specify a region of the sky by selecting an object (planet, star, constellation, etc.) and a radius.  When applied only those passes that come within the "radius" distance of the object are shown.  These passes are still in memory, so you can show them again by toggling the filter off and on.  To use the cone filter select Controls|Set Cone Parameters from the popup menu.

The Multiple Skytraces window is extremely valuable if you are using it on a laptop computer.  You can set the window to run in full screen mode (select Full Screen from the popup menu) and then animate it in real time.  When you set your computer up outside, you can see the satellites moving, on screen, in a way that it exactly matches the motion you see in the sky.  To help with your viewing, you can set the screen colors to the pre-defined Night Sky scheme.  This helps preserve your night vision when looking at the screen and then back at the sky.  To set the Night Sky colors go to Options|Colors and click on the button labeled Night Sky.  While you are watching for satellites, you can log the satellites you are successful in observing.  Select Controls|Log Sightings… from the popup menu.  This puts SatSpy into a logging mode.  When you see a satellite, use the mouse to click on the corresponding pass.  You will hear a beep and see a popup message showing the time and positional information.  In addition, the same information will be written to a log file (ObsLog.txt, by default) for later analysis.  On a clear, dark night, you will be amazed at how many satellites you can see using this technique.

For more information on using the Multiple Skytrace window see the following topics in the online help:

- Multiple Skytraces

- Multiple Skytrace Detail

- Animating Graphical Windows

- Logging Sightings

- Using the Cone Filter

- Changing the Color Scheme

- Elset Filter Page  - How to set and use elset filtering

- Pass Quality Pages - How to set and use pass quality

- Compute Viewing Opportunities Dialog

The Viewing Opportunities Window

The Viewing Opportunities window compliments the Multiple Skytraces window by showing a set of passes as a timeline chart.  Where the former window shows you how the passes appear in the sky, this window shows you the dynamics of the passes.  Each pass is shown as a bar on a horizontal chart.  The name of the satellite is shown on the left side of the chart and the estimated brightness at culmination is on the right. The length of the bar is directly proportional to the duration of the pass.  The bars are color coded to indicate the quality of the pass, using criteria that you can specify.  By default, passes are colored green, yellow and red to indicate the best passes to the worst, respectively.  The best passes are those that are relatively high in the sky and directed away from the Sun.  The worst are those that are low to the horizon and looking toward the Sun.  To help in you planning, the times of sun rise/set and the start and end of civil, nautical and astronomical twilight are shown as vertical lines.  When a large number of passes are displayed, a vertical scroll bar will appear on the right side of the window.  This allows you to scroll the passes up and down while keeping the caption lines fixed at the top and bottom.

By default, all passes are shown and are sorted in ascending time from top to bottom.  You can change the sort order from the Controls command on the popup menu.  The passes can be sorted by satellite name, satellite number, pass quality, rise time and visual magnitude.  The latter option is especially useful for finding the brightest passes.  You can filter passes to exclude daylight and shadow passes and to limit the visible passes by quality.  This is done from the Filter page of the SatSpy Preferences dialog (Options|Elset Filter… from the main menu).  You can also filter passes using the as described above for the Multiple Skytraces window.  You can get additional information on any pass by clicking to open the Viewing Opportunity Detail dialog as described above.

The Viewing Opportunities window is the best tool for doing relatively long range pass prediction.  You can predict up to a month in advance with this tool.  Beyond that, the nature of the elset data is such that the predictions will probably not be accurate.  When you do long range predictions, you will typically see multiple passes for the same satellite.  You can use the compressed mode (Controls|Compressed Mode from the popup menu) so that all passes for a single satellite will show up on a single line.  Alternatively, you can zoom in on a subset of the passes using the Zoom command.

To open a new Viewing Opportunities window, select Sat Views|Viewing Opps… from the main menu or use the toolbar.   This opens the Compute Viewing Opportunities dialog, from which you pick one or more satellites along with a time span of interest.  You can use the Filter button to apply constraints to the passes you wish to view.  When you click OK, the progress bar opens in the lower right-hand side of the main window and indicates the progress of the computations.  When complete, you see the Viewing Opportunities window open showing the passes available during the time span.

The Viewing Opportunity window is designed to work with the Multiple Skytrace window as well as a standalone window.  You can open either window as a "child" of the other from the View command in the popup menu.  This gives two complimentary views of the same set of passes.  Opening a child Viewing Opportunity window is especially useful when animating the Multiple Skytrace window.  At each step in the animation, a message is sent from the Multiple Skytrace window to the Viewing Opportunity window.  When the latter window receives the message, it draws a vertical, red line showing the current time in the multitrace window.  This give a quick visual cue to indicate which passes are currently visible and what passes are coming up.

For more information on using the Viewing Opportunity window see the following topics in the online help:

- Viewing Opportunities Window

- Viewing Opportunities Detail

- Using the Cone Filter

- Changing the Color Scheme

- Elset Filter Page  - How to set and use elset filtering

- Pass Quality Pages - How to set and use pass quality

- Using the Compressed Mode

- Compute Viewing Opportunities Dialog

The Geosynchronous Satellites Window

The windows discussed up to this point are used to predict visual passes for near earth satellites.  They are not very useful for viewing satellites in deep space orbits.  The Geosynchronous Satellites window fills this gap and allows you to predict where to look for satellites in orbits that keep them in fixed positions relative to the earth's surface.  As such, the satellite's positions are shown as fixed points in the sky rather than as traces. You can use this window to find out where your DSS satellite is located as well as tracking weather satellites.  If you have a powerful telescope, you might actually be able to see one.

SatSpy distinguishes between satellites that are "geostationary" and nearly so.  A geostationary satellite has an orbit that, in conjunction with active "station keeping" by its owner, keeps it in a fixed location.  A near geostationary satellite will tend to drift about a fixed point in the sky usually in a figure eight motion caused by its inclination. The satellite's positions are shown as color coded symbols, red for satellites in geostationary orbits and yellow for near-geostationary orbits.  The definitions of which is which is set in the Sync Sats page of the SatSpy Preferences dialog.  Only satellites which satisfy the criteria set on this page are displayed in the Geosynchronous Satellite window.  This window shares the same basic functionality as the other local sky windows (Single Skytrace, Local Sky, etc.).

Before opening a Geosynchronous Satellite windows, you need to be sure that the elsets that you wish to view are loaded using the SGP4/SDP4 orbit propagator.  This is a special propagator type that models some of the unique aspects of a geosynchronous satellite orbit.  Select Options|Elsets from the main menu to open the SatSpy Preferences dialog at the Elsets page.  From there, click on the SGP4/SDP4 button under the Theory Type group.  You must do this before loading the elsets, otherwise you will see a warning message when you open the window.

To open a Geosynchronous Satellite window, select Sat Views|Synch Sats… from the main menu or the toolbar.  You will then see the Pick a Satellite for Geotrace dialog showing a list of all satellites that meet the geostationary criteria.  If you don't see any names listed, you will need to load a new elset file containing geostationary satellite elsets.  When you do see names listed, you will notice that some of them are gray and some are black.  The black names are for satellites that are visible from your viewing location at the current time.  The gray names are for satellites that are below your local horizon.  You can pick individual satellite(s) from the list or click on the box at the bottom of the list to select them all.  Once you have made your selection, click on the OK button to open the window.

For more information on using the Geosynchronous Satellite window see the following topics in the online help:

- Geosynchronous Satellite Window

- Geosynchronous Satellite Window Detail

- Animating Graphical Windows

- Geo Tabular Data

Supporting Graphical Data Windows

SatSpy provides a several supporting graphical data windows to help you understand the geometry of a pass or to experience another view of the sky.  These include the Groundtrace and the 3D Orbit View windows, which give you a 2-dimensional and 3-dimensional representation, respectively, of a satellite's orbit.  Both windows can be opened as "child" windows of the Single Skytrace and other windows.  When the parent skytrace is animated, the motion of the satellite can be observed in the child window as well.

The Local Sky and Right Ascension/Declination windows, give you additional ways to look at the stars, planets and constellations helping you to orient yourself for more successful viewing.

A.The  Groundtrace Window

The Groundtrace window shows the position of one or more satellites as each moves through one or more orbits (revs).  When the window opens, the groundtrace is shown for a single revolution for the selected satellite(s).  Each trace consists of a yellow, usually sinusoidal, line with red tic marks showing the ephemeris sub-points.  The position of the satellite at the current time is indicated by a red cross surrounded by the satellite's footprint.  The groundtrace is projected on a map showing continental coastlines and major islands.  The day and night side of the earth are indicated by light and dark blue regions, respectively.  The boundary lines for civil, nautical and astronomical twilight are shown as light blue lines in the night portion of the map.  The position of the sub-point of the sun is shown in the middle of the day side of the earth.  The current viewing location is shown as a cross labeled with the name of the site.  The status panel along the bottom shows the current position of the satellite to the left.  The right hand side of the panel shows the latitude and longitude of the point under the mouse cursor.  Notice that this changes as you move the mouse.  (Tip:  You can change the latitude/longitude format by double clicking on the panel.)

To open a Groundtrace window, select Orbit Views|Groundtrace… from the main menu or from the toolbar.  If you have not already done so, you are prompted to load the element set list.  The Pick a Satellite for Groundtrace dialog opens, showing the names of all satellites currently loaded into memory.  Use the mouse to pick one or more satellites for viewing and then click on the OK button.  The Groundtrace window opens showing the satellite(s) that you selected.  By default, the time is set to the current local time from your system clock.  Alternatively, you can open the Groundtrace window from many of the other graphical windows by selecting View|Groundtrace  from the popup menu.  When you use this mechanism, the satellite(s) showing in the parent window appearsin the child Groundtrace window at the time taken from the parent.

You can control much of what is displayed in the Groundtrace windows from the Groundtrace Viewing Controls dialog.  To open this dialog, select Controls|Controls… from the popup menu or press the F4 function key.  This dialog consists of a four page (Trace, Toggles, Day/Night and Time) tabbed notebook and a number of buttons.  The Trace page allows you to control the appearance of the groundtrace and the display of the satellite's "footprint".  The Toggles page allows you to turn many of the display items, including the latitude and longitude grid, off and on.  The Day/Night page controls the display of the day and night portions of the map along with the three twilight lines.  Finally, the Time page allows you to change the position of a satellite in its orbit by changing the display time.  Once you have made your changes, you can click on the Show button to redraw the window to reflect the changes.  When you are satisfied, click on the OK button to close the dialog.

The Groundtrace window can be animated, zoomed, printed, saved, etc. much like the other graphical windows.  For more information, see the following topics in the online help:

- Groundtrace Window

- Groundtrace Window Detail

- Animating Graphical Windows

- Satellite Footprint

- Groundtrace Viewing Controls

- Displaying Multiple Satellites

The 3D Orbit View Window

The 3D Orbit View window compliments the Groundtrace window by showing a satellite's orbit in 3D.  As in the Groundtrace window, you can show the orbits of one or more satellites simultaneously.   The satellite's orbit is shown as an ellipse around and above the figure of the spherical Earth or as a groundtrace projected onto the surface of the sphere.  The day side of the earth is shown in light blue while the night side is colored navy blue.  The boundary lines for civil, nautical and astronomical twilight are shown as light blue lines, parallel to the terminator, on the night side of the earth. The Sun's subpoint is shown on the day side of the earth.  The orbit representation is color-coded to indicate when the satellite is in daylight and when it is in the earth's shadow.  The portion of the orbit that is in daylight is shown in yellow while the red color shows the part that is going through the earth's shadow.   When more than one satellite is showing, one of those satellites is the "primary" satellite.  The primary satellite has special properties which are described below.

The status panel at the bottom of the window contains additional information about the display.  The status panel consists of three panes.  The pane on the left shows the current position of the primary satellite in its orbit.  This pane will automatically update when you change the time or during animation.  If you double-click on the pane, the display switches to show the time, azimuth, elevation and range from the viewing location to the primary satellite.  This is useful during animation.  The middle pane in the panel shows the current Viewpoint (described below).  The right most pane shows the current primary satellite.  If more than one satellite is showing, you can double click on this pane to open the Pick New Primary Sat dialog from which you can change the current primary satellite setting (You can also change the primary by selecting Controls|Primary Sat from the popup menu).

The satellite orbit and the figure of the Earth are shown as if you were looking down from an imaginary point in space, called the Viewpoint.  This window provides a number of predefined Viewpoints that are accessible from the Viewpoints page of the 3D Viewing Controls dialog.  The predefined viewpoints are described in the online help.  Many of the predefined viewpoints are derived from the orbital characteristics of the satellite being displayed.  When more than one satellite is showing, the viewpoints is always referred to the current primary satellite.  In addition to the predefined points, you can arbitrarily set the viewpoint to any point in space by specifying a latitude, longitude and zoom factor.

Other pages in the 3D Viewing Controls dialog allow you to adjust the orbit and footprint display (Orbits), turn various display items off and on (Toggles), control the display of day and night (Day/Night) and adjust the position of the satellite(s) in their orbits (Time).  To open the 3D Viewing Controls dialog, select Controls|3D Controls… from the popup menu or press the F4 function key.  Alternatively, you can double click on the Viewpoint pane in the status panel.  This opens the dialog at the Viewpoints page.

The 3D Orbit View window can be animated, zoomed, printed, saved, etc. much like the other graphical windows.  For more information, see the following topics in the online help:

- 3D Orbit View

- 3D Orbit View Detail

- Animating Graphical Windows

- Satellite Footprint

- 3D Viewing Controls

- Displaying Multiple Satellites

- Zooming the 3D Orbit View

- Setting the Viewpoint

- 3D Orbit Trace Types

Local Sky Window

The Local Sky window shows the sky as it appears in the local horizon coordinate system from your latitude and longitude.  The plot includes all of the bright stars and major constellations, the sun and moon and the seven of the nine planets (Mercury, Venus, Mars, Jupiter, Saturn, Uranus and Neptune).  The ecliptic (the path of the Sun across the sky) is shown as a red line running across the sky, generally through the constellations of the Zodiac.  Each of the planets and the moon may be found near the ecliptic.  This window supports all of the functions described above for star windows.

1.To open a Local Sky window, select Star Views|Local from the main menu or toolbar.  You do not have to have any elsets loaded to use this window.  When the window opens, the current local time is taken from your system clock and the sky is shown at that time.  You can change the time by selecting Controls|Time from the popup menu or by pressing the F2 function key.

The Local Sky window supports animation and  zooming like all other SatSpy graphical windows.  The animation feature is especially interesting for viewing the motion of the Sun and planets over long periods of time.  You can also use this window to view the Cone Filter settings (the projection of the imaginary cone against the sky) before applying them to a Multiple Skytrace or Viewing Opportunity window.

For more information on using this window, see the following topics in the online help:

- Local Polar

- Local Polar Window Detail

- Using the Cone Filter

- Animating Graphical Windows

- Zooming In and Out

- Celestial Controls

- Sun Rise and Set Times Dialog

Right Ascension/Declination Window

The Right Ascension/Declination window shows the entire celestial sphere plotted in Cartesian format.  The plot includes all of the bright stars and major constellations, the sun and moon and the same seven as described above.  The ecliptic is shown as a red line running across the sky, generally through the constellations of the Zodiac.  Each of the planets and the moon may be found near the ecliptic.  The status panel along the bottom of the window shows the position (right ascension/declination) of the point under the cursor.  As you move the mouse, you will see the coordinates in the position panel continually updated.  You can change the display format by double clicking on the panel.

To open a Right Ascension/Declination window, select Star Views|Ra/Dec from the main menu or toolbar.  You do not need elsets to use this window.  When the window opens, the current local time is taken from your system clock and the sky is shown at that time.  You can change the time by selecting Controls|Time from the popup menu or by pressing the F2 function key.

As with the Local Sky window, the animation feature is especially interesting for viewing the motion of the Sun and planets over long periods of time.  You can also use this window to view the Cone Filter settings (the projection of the imaginary cone against the sky) before applying them to a Multiple Skytrace or Viewing Opportunity window.

For more information, see the following topics in the online help:

- RaDec Star Plot

- RaDec Plot Detail

- Using the Cone Filter

- Setting the View Time

- Animating Graphical Windows

- Celestial Controls

- Zooming In and Out

Supporting Tabular Data Windows

SatSpy provides a number of supporting tabular data windows.   The windows can be broken down into three classes:  ephemeris data windows, the elset summary window and text editing windows.  The ephemeris data windows all provide positional information for a variety of objects at uniform time steps over a time span you specify.  These windows are not interactive, but you can select and copy information to the clipboard and send the contents of the window to a text file or the printer.  The Elset Summary window shows a sorted summary of the elset list.  The text editing window is a fully functional text editor which you can use to edit elsets or any other text file.

Each of the available windows are described below.  For more information, see the related topics in the online help files listed at the end of this chapter.

The Elset Summary Window

The Elset Summary window presents a condensed version of the current contents of the elset list and includes the following fields:  satellite number, common name, international designator, period, eccentricity, inclination, right ascension of the ascending node (RAAN), epoch age and visual magnitude.  By default, the summary is sorted by satellite number.  You can change the sort order by clicking on the header button for the desired sort column.  Clicking on the same button two times in succession changes the sort order from ascending to descending or vice versa.  You can search for a particular satellite by using the find command.  This is accessible from the main menu, the popup menu or from the keyboard (ctrl-F).  The contents of the window can be saved to file (Save ctrl-s) or sent to the printer (Print) from the popup menu or main menu.

This window can also be used to open other SatSpy windows.  When a satellite is selected (by clicking anywhere on the summary line), the popup menu contains commands for opening a Single Skytrace, Groundtrace, 3D  Orbit View or Elset Detail window.  If the satellite you selected is a geostationary satellite (as described above), the popup menu also contains a Sync Sat command.  Selecting any of these commands will open the corresponding window using the current local time from your system clock.

To open an Elset Summary window, select Elset Views|Sorted Summary from the main menu or from the toolbar.  For more information on this window, see the following topics in the online help:

- Viewing Elset Summaries

- Viewing Elset Summary Detail

- Open Supplementary Data Windows

- Change the Sort Order

The Look Angles Window

The Look Angles window gives you the option to compute positional information for a satellite relative to the current viewing location.  At each time step, three data groups are displayed: azimuth, elevation and range from the viewing location to the satellite; the right ascension and declination of the satellite projected against the celestial sphere; and the latitude, longitude and altitude of the satellite's subpoint.  The contents of the window can be copied to the clipboard, saved to a text file or printed.

You can open a new Look Angles window by clicking on Eph. Tables|Look Angles from the main menu.  If you have not already done so, you will be prompted to open an element set file.  The Compute Look Angles dialog is opened prompting you to select a satellite and  to specify a start time, a time span and a time step.  (Be careful when setting the time step and time span.  If you use a large span and a small step, the amount of data may be such that some lines are truncated.  If this happens, try shortening the time span.) When you click on the OK button, the window will open showing the  results of the look angles calculations

The Satellite Ephemeris Window

The Satellite Ephemeris window shows the position and velocity ephemeris of a single satellite.  Both the position and velocity are given as Cartesian coordinates (x,y,z) in the inertial (ECI) coordinate frame.  In addition, the magnitude of the position and velocity vectors are also given.  The magnitude of the position vector is the distance from the center of the Earth (the origin of the ECI coordinate frame) to the satellite.  The magnitude of the velocity vector gives the instantaneous velocity of the satellite.  The units are defaulted to Km and Km/Sec.  You can change the display coordinates from the Units page of the SatSpy Preferences dialog (Options|Units).  The contents of the window can be copied to the clipboard, saved to a text file or printed.

You can open a new Satellite Ephemeris window by clicking on Eph. Tables|Sat. Ephemeris from the main menu.  If you have not already done so, you will be prompted to open an element set file.  The Compute Satellite Ephemeris dialog is opened prompting you to select a satellite and  to specify a start time, a time span and a time step.  (Be careful when setting the time step and time span.  If you use a large span and a small step, the amount of data may be such that some lines are truncated.  If this happens, try shortening the time span.) When you click on the OK button, the window will open showing the satellite's ephemeris.

The Planetary Ephemeris Window

The Planetary Ephemeris window shows the position ephemeris of a planet or the Sun or Moon.  The position is given, at each time step, as a right ascension/declination value.  No range value is computed.  The right ascension and declination values give the position of the celestial object as viewed from the Earth (again in the ECI coordinate frame).  The positions are referenced to the J2000 epoch.

You can open a new Planetary Ephemeris window by clicking on Eph. Tables|Planet Ephemeris from the main menu.  You do not need satellite elsets for this computation.  The Celestial Object Ephemeris dialog is opened prompting you to select a planet or the Sun or Moon and  to specify a start date, an end date and a time step.  Given the relatively slow movement of these objects, the time step can only be specified in days. When you click on the OK button, the window will open showing the celestial object's ephemeris.

The Relative Motion Analysis Window

The Relative Motion Analysis window presents data showing the positional information on two satellites simultaneously.  This includes the ECI coordinates of both position vectors as well as the distance between them.  The distance is given both in terms of a "slant range" distance and an angular distance.  The slant range distance is the absolute number of kilometers (or other units) separating the two objects.  The angular distance is the instantaneous angle between the two position vectors.

This window is especially useful when you observe two satellites passing near each other and you want to know how close they actually came.  To open a Relative Motion Analysis window, select Eph. Tables|Relative Motion from the main menu to open the Relative Motion Analysis dialog.  From here, select the names of the two satellites you saw and then set the start time and time span to bracket the time you saw them.  Click on OK to open the window.  You may need to adjust the time span and time step values to get the actual time of closest approach.

The Geosynchronous Satellite Ephemeris Data Window

The Geosynchronous Satellite Ephemeris Data window shows the position of satellites matching the geosynchronous satellite criteria described above.  Unlike the other ephemeris data windows, this window shows the positions of several different satellites at a single point in time.  The positional information includes the azimuth, elevation and range to each satellite from the viewing location, the right ascension and declination and the satellite's latitude and longitude.  The data points are sorted, in increasing order, by longitude.  The longitude is given as the east longitude which varies from 0 to 360 degrees, going east from the prime meridian.

To open a Geosynchronous Satellite Ephemeris Data, select View|Tabular Data|Visible Sats or View|Tabular Data|All Sats from the popup menu.   The first option shows only those satellites that are visible from the current viewing location.  The second option lists all satellites which match the geosynchronous criteria, regardless its visibility.

The Skytrace Ephemeris Data Table Window

The Skytrace Ephemeris Data Table contains the time for each trace step along with:  the azimuth, elevation and slant range from the observer to the satellite; the right ascension and declination showing the projection of the line of sight from the observer though the satellite onto the celestial sphere;  the percent illumination of the satellite; and the estimated visual magnitude.  When the percent illumination is set to zero, the satellite is in the earth's shadow.  The corresponding value for the estimated visual magnitude is set to '(Shad)'.  All angles are given in degrees while distances are given in kilometers.

This window is only available from the Single Skytrace graphical window. To open a Skytrace Ephemeris Data Table, select View|Ephemeris Data from the Skytrace window's popup menu.

The Viewing Opportunity Table Window

The Viewing Opportunity Table window shows a one-line summary of viewing opportunities (passes).  These opportunities are computed in a parent window and passed to this window when it is opened.  The parent can be any one of the following windows:  Single Skytrace, Viewing Opportunity and Multiple Skytrace.  The data is presented as columns including:

Items pertaining to the entire viewing opportunity.  All times are local.  Rise and set are the times when the satellite crosses the minimum elevation angle specified in the SatSpy Preferences dialog.

- Satellite number

- Common name for the satellite

- Date of the pass

- Rise time

- Culmination time (when the satellite reaches max. elevation)

- Set time

Items pertaining to the pass culmination

- Azimuth, elevation and range.

- Percent illumination.

- Estimated visual magnitude

- Pass quality (Grn, Yel, Red)

- Estimated visual magnitude of the satellite.

To open a Viewing Opportunities Table, first open a Single Skytrace, Multiple Skytrace or Viewing Opportunities window containing the passes of interest.  Then, select View|Opportunity Table from the popup menu.  If you open a Multiple Skytrace window as a child of a Viewing Opportunity window, or vice versa, the Viewing Opportunities Table contents will be the same for each window.  If the parent is a Single Skytrace window containing a single pass, the opportunity table is not available.  Once opened, this window shares the properties (print, save to file, etc.) of the other tabular data windows.

The Text Editing Window

The Text Editing window gives you the capability to open, edit and save text files.  It is primarily provided to allow you to work with elset files, but you can use it to manipulate any kind of text file.  You can also create new text files from SatSpy.  This is useful when you want to cut and paste elsets from different files to create your own custom elset file.

If you edit an element set, save it to a file and then try to load it into the elset list, it will probably be rejected because of a checksum failure.  To get around this, turn off the Verify Checksum option from the Toggles page in the SatSpy Preferences dialog.  Keep in mind also, whenever you change an element set, you are changing the model of the orbit and not the orbit itself.  If you use a modified elset for pass prediction, it will fail.

To open a Text Editing window for an existing file, select File|Edit File... from the main menu.  This brings up the Specify Elset File dialog showing all files in the current directory with the elset file extension specified by your preferences.  You can use the dialog box to open any file you wish, as long as it is a text file.  Once you have chosen a file, click on the OK button and the window opens.  The caption shows the name of the file you selected

To create a new file, select File|New File from the main menu.  An empty Text Edit window opens.  You can enter text manually or copy text from the clipboard.

For more information on these windows, see:

- Satellite Ephemeris Window

- Changing Units

- Relative Motion Analysis Window

- Changing Units

- Opening a Relative Motion Window

- Geosynchronous Satellite Tabular Data

- Skytrace Ephemeris Table

- Skytrace Ephemeris Data Detail

- Look Angles

- Planetary Ephemeris Window

- Viewing Opportunity Table

- Viewing Opportunity Table Detail

- Text Editing Window

- Text Editing Detail

What is an Element Set?

An element set is a collection of parameters that are used, along with an orbit propagator, to predict the motion of a satellite.  The element set, or elset for short, consists of identification data, the classical elements and drag parameters.  A typical two-line element set is shown below:

Several of the fields, such as satellite number and Intl. Desig. (also known as COSPAR number) identify the satellite.  The line number, element number and rev. number identify the particular elset and its components.  The check sum is a number that is added at the sending site to ensure the integrity of the data during transmission.  This number is (optionally) checked by SatSpy when loading new elsets.  The remaining fields contain the actual data used by SatSpy to predict the satellites motion.

For a field-by-field description of a typical elset see:

- What is an Elset?

- Line 1 Fields

- Line 2 Fields

- Checksum

Finding Fresh Element Sets

Element sets are available from a variety of sources on the Internet and the commercial on-line services.  Since the release of version 1.0, the use of the Internet, specifically the World Wide Web (the web for short), has exploded.  Several sources are shown below.   For a more complete description of elset sources, see the on line help files.  Keep in mind that the web is very dynamic entity.  Sites and sources are constantly changing.  The only up to date information about the web is the on web itself.

Acappella Publishing Web Site

Acappella Publishing maintains a website for SatSpy at http://www.satspy.com/.  You can always find the latest information about SatSpy at this location.  In addition, the home page contains links to all of the sites on the Internet where you can find updated element sets.

Internet FTP

There are many ftp sites on the Internet which carry orbital element sets.   Some sites carry elsets that are specifically selected for viewing while other contain elsets for many more satellites that are not necessarily good viewing targets.  To access these sites, log on using "anonymous" when prompted for a user-id and your e-mail address when prompted for a password.  Some of the best sites are:

1.  Celestial Archives - (ftp.celestrak.com)

2.  NASA JPL - (kilroy.jpl.nasa.gov)

3.  University of Arizona - (seds.lpl.arizona.edu)

World Wide Web

The World Wide Web (or simply, the web) is probably the best source for your satellite watching needs.   There are many sites where you can find orbital element sets and other information about satellites and satellite viewing.  In fact, there are so many, that it would be fruitless to try to mention them all.  I will list some of my favorites below.  Keep in mind, however, that the web is constantly changing and the URLs listed may change.  Some choice sites include:

1.Visual Satellite Observers Homepage - This site is probably the best single site for satellite viewing.  It contains a lot of very interesting information and tips on satellite viewing as well as a thorough set of links to other, related sites.  (http://www2.satellite.eu.org/sat/vsohp/satintro.html)

2.Celestial BBS on the Internet - Contains many of the same files that can be found on the Celestial Archives ftp site.  This is one of the best sites for elsets. (http://celestrak.com/)

3.Orbital Information Group's Web Page - This is the primary source for all "official" element sets. When you download elsets from this source, you may experience problems loading the elsets.  Some browsers do not include the carriage return and line feed (CR/LF) characters expected by DOS-based programs such as SatSpy.  If you are a registered user of SatSpy, you can use the SatSpy Utilities to insert CR/LF characters.  If you have not yet registered, try loading the elset into a DOS editor (such as Edit) and then saving them back to the file. (http://oigsysop.atsc.allied.com/)

4.Satellite Observing Resources - This site contains references to Internet resources on artificial satellite tracking.  (http://www-leland.stanford.edu/~iburrell/sat/sattrack.html)

Telnet

You can also access the NASA Orbital Information Group (OIG) via the internet Telnet protocol.  From there you can find shuttle vectors and element sets and data for most other objects in the USSPACECOM catalog.  To access this site, you must have Telnet client software along with access to the Internet.  Use the following procedure:

1.Telnet to oig1.gsfc.nasa.gov.

2.When prompted for a login ID enter 'oig'.

3.When prompted for a password, enter 'goddard1'.

4.Follow the instructions for a simple registration process and then you can download the data you desire.

Online Services

This group includes the commercial online service providers such as CompuServe, Prodigy and America Online (AOL).  CompuServe provides element sets posted in the Sats and Sky Viewing library in the Astronomy Forum (GO ASTROF).  I do not know if the other services provide element sets.  To find out, check out their astronomy sections.

Technical Support

Non-registered users

Support is limited to help in installing and running the program to the extent that it can be properly evaluated.  Full product support is one of the benefits of registration.

Registered users

Registered users are entitled to full product support including:

- Setup assistance.

- Assistance concerning the operation or usage of SatSpy.

- Assistance in problem resolution.

- Bug-fixes will be made available, free of charge, on an "as needed" basis.

- Acappella Publishing reserves the right to determine when fixes are necessary.

How to request assistance

You can contact Acappella Publishing by any one of the means described below.  We will make every attempt to provide you with a response within 24 hours of our receipt of your message.  We check all of our contact points on a daily basis, except on weekends and US holidays.  When you request assistance, the following information is required:

1.  Your name, address, and registration number (provided when you register with us).

2.  A concise description of the nature of your request.  Be as complete as possible.

If you are reporting a problem, the following information will help us in resolving the

problem:

1. Your computer

a) Computer make and model

b) CPU type (e.g., 386,486,Pentium)

c) How much memory (e.g., 4Meg, 8Meg, etc.)

d) Windows/DOS version numbers

e) Printer type

f) Any other information you may feel is pertinent.

2. A description of what you were doing when the problem occurred.

a) Which menu command or button was involved?

b) Type of the last window in use (e.g., Skytrace, Groundtrace, etc.)

c) The text of any error messages that were displayed.  If an address

(e.g., 0001:01C3) is displayed, try to record it before clearing

the message.

How to Contact Me:

E-mail:

support@satspy.com

71672.1356@compuserve.com

dcappell@rmii.com

dcappell@usa.net

Postal Service:

Acappella Publishing

P. O. Box 50126

Colorado Springs, CO  80949-0126

CompuServe:

Send mail to 71672,1356.

We also monitor the messages posted to the Viewing Satellites group

in the astronomy forum.

World-Wide Web:

SatSpy Home Page:  http://www.satspy.com/